## Objective

The objective of reverse engineering the flame sensor module is to identify all the electronic components used on the PCB, understand their functions, analyze the circuit connections, and explain how the module detects a flame.

## Components Used

### 1. Flame Sensor (IR Photodiode / IR Receiver)

- This is the main sensing element.

- It detects infrared (IR) radiation emitted by a flame.

- Most flames emit IR light in the range of approximately 760 nm to 1100 nm.

- The sensor converts the received IR light into an electrical signal.

### 2. LM393 Comparator IC

- The LM393 is a voltage comparator.

- It compares the sensor output voltage with a reference voltage.

- When the sensor voltage crosses the reference level, the comparator changes the digital output state.

- It provides a stable HIGH or LOW signal for a microcontroller.

### 3. Potentiometer (Variable Resistor)

- Used to adjust the flame detection sensitivity.

- It changes the reference voltage applied to the comparator.

- Rotating the potentiometer increases or decreases the detection distance.

### 4. Indicator LEDs

Power LED

- Indicates that the module is receiving power.

Detection LED

- Turns ON or OFF when a flame is detected.

### 5. Resistors

- Limit current through the LEDs.

- Provide biasing for the comparator.

- Form voltage divider networks where required.

### 6. Capacitors

- Filter power supply noise.

- Improve circuit stability.

- Reduce false triggering caused by electrical noise.

### 7. Header Pins

- VCC – Power supply (3.3V or 5V)

- GND – Ground

- DO (Digital Output) – Digital flame detection output

- AO (Analog Output) – Analog sensor output (available on some modules)

# Working Principle

- The module receives a power supply through the VCC and GND pins.

- The flame sensor continuously detects infrared radiation.

- When a flame is present, the IR intensity increases.

- The sensor converts the IR light into a voltage signal.

- The LM393 comparator compares this voltage with the reference voltage set by the potentiometer.

- If the sensor voltage exceeds the reference level, the comparator changes the digital output state.

- The detection LED turns ON (or OFF depending on the module design).

- A microcontroller such as Arduino, ESP32, or STM32 reads the digital or analog output to determine whether a flame is present.

Component Summary

| Component | - Function |
| --- | --- |
| - Flame Sensor | - Detects infrared radiation from a flame |
| - LM393 Comparator | - Compares sensor voltage with reference voltage |
| - Potentiometer | - Adjusts detection sensitivity |
| - Resistors | - Current limiting and biasing |
| - Capacitors | - Noise filtering and power stabilization |
| - LEDs | - Power and flame detection indication |
| - VCC/GND | - Power supply connections |
| - DO | - Digital output |
| - AO | - Analog output (if available) |