## Objective

- To identify all the components used in the MQ Gas Sensor module.

- To understand the function of each component.

- To redraw the schematic in KiCad.

- To study the working principle of the module.

# Reverse Engineering of MQ Gas Sensor Module – Components and Their Functions

## 1. MQ Gas Sensor (Main Component)

### Function

- Detects the concentration of gases such as LPG, Methane, Smoke, Hydrogen, Alcohol, Ammonia, Carbon Monoxide (depends on sensor type).

- Changes its resistance based on the gas concentration.

### Why it is used

- To detect the presence and concentration of gases.

## 2. Heating Element (Inside MQ Sensor)

### Function

- Heats the sensing material to its operating temperature.

### Why it is used

- Enables the gas-sensitive layer to react with gases.

## 3. Tin Dioxide (SnO₂) Sensing Element (Inside MQ Sensor)

### Function

- Changes resistance when exposed to target gases.

### Why it is used

- Acts as the gas sensing material.

## 4. LM393 Comparator IC

### Function

- Compares the sensor output voltage with a reference voltage.

- Produces a digital HIGH or LOW output.

### Why it is used

- Provides digital gas detection.

## 5. Preset Potentiometer (Typically 10 kΩ)

### Function

- Adjusts the gas detection threshold.

### Why it is used

- Allows the user to set the desired sensitivity.

## 6. Load Resistor (RL)

### Function

- Forms a voltage divider with the sensor resistance.

### Why it is used

- Converts resistance changes into measurable voltage.

## 7. Power Indicator LED

### Function

- Indicates that the module is powered.

### Why it is used

- Shows power status.

## 8. Output Indicator LED

### Function

- Turns ON when gas concentration exceeds the threshold.

### Why it is used

- Indicates gas detection.

## 9. LED Current Limiting Resistors

### Function

- Limits current through the LEDs.

### Why it is used

- Protects the LEDs.

## 10. Decoupling Capacitor (Typically 100 nF)

### Function

- Filters power supply noise.

- Stabilizes the operating voltage.

### Why it is used

- Improves reliable operation.

## 11. VCC Pin

### Function

- Supplies 5V power.

### Why it is used

- Powers the module and heater.

## 12. GND Pin

### Function

- Provides the common ground reference.

### Why it is used

- Completes the electrical circuit.

## 13. AO (Analog Output)

### Function

- Outputs an analog voltage proportional to the gas concentration.

### Why it is used

- Enables gas concentration measurement.

## 14. DO (Digital Output)

### Function

- Outputs HIGH or LOW depending on the comparator result.

### Why it is used

- Enables threshold-based gas detection.

# Pin Description

| Pin | Function |
| --- | --- |
| VCC | +5V Power Supply |
| GND | Ground |
| AO | Analog Output |
| DO | Digital Output |

# Working Principle

- The heating element warms the SnO₂ sensing layer.

- Gas molecules come into contact with the sensing material.

- The sensor resistance changes according to the gas concentration.

- The voltage divider produces a varying analog voltage.

- The AO pin outputs this analog voltage.

- The LM393 comparator compares the voltage with the threshold set by the potentiometer.

- If the threshold is exceeded, the DO pin changes state.

- The output LED indicates gas detection.

# Signal Flow

Gas

↓

MQ Gas Sensor

↓

SnO₂ Sensing Element

↓

Voltage Divider (RL)

↓

AO (Analog Output)

OR

Voltage Divider

↓

LM393 Comparator

↓

DO (Digital Output)

↓

Arduino / ESP32 / STM32

# Typical Components on MQ Gas Sensor Module

- MQ Gas Sensor (MQ-2 / MQ-3 / MQ-5 / MQ-135, etc.)

- Heating Element (inside sensor)

- SnO₂ Gas Sensing Element (inside sensor)

- LM393 Comparator IC

- 10 kΩ Potentiometer

- Load Resistor (RL)

- Power LED

- Output LED

- LED Current Limiting Resistors

- 100 nF Decoupling Capacitor

- Pin Header

# Applications

- LPG Gas Leakage Detection

- Smoke Detection

- Fire Safety Systems

- Air Quality Monitoring

- Industrial Gas Monitoring

- Smart Home Automation

- IoT Gas Monitoring

- Environmental Monitoring

- Embedded Systems

- Robotics

# Advantages

- Detects multiple gases (depending on the MQ sensor model)

- Provides both analog and digital outputs

- Adjustable sensitivity

- Easy to interface with Arduino, ESP32, and STM32

- Low cost and widely available

- Suitable for indoor gas monitoring

- Reliable for gas leak detection