TITLE

Reverse Engineering of IR Obstacle Sensor Module

1. IR LED (Infrared Transmitter)

Function:

Emits infrared (IR) light continuously toward the sensing area.

Why it is used:

To transmit IR light that can be reflected by nearby objects.

2. IR Receiver (Photodiode / Phototransistor)

Function:

Detects the reflected infrared light from an object.

Converts the received light into an electrical signal.

Why it is used:

To sense the presence of an object.

3. LM393 Comparator IC

Function:

Compares the signal from the IR receiver with a reference voltage.

Produces a clean digital HIGH or LOW output.

Why it is used:

To provide a stable and reliable digital output for the microcontroller.

4. Potentiometer (Typically 10 kΩ)

Function:

Adjusts the reference voltage of the comparator.

Changes the sensing distance and sensitivity.

Why it is used:

To calibrate the detection range according to the application.

5. Current Limiting Resistors

Function:

Limit the current flowing through the IR LED and indicator LED.

Protect the LEDs from excessive current.

Why it is used:

To ensure safe operation and increase component lifespan.

6. Indicator LED

Function:

Turns ON when an object is detected.

Indicates the sensor status visually.

Why it is used:

To provide an easy indication of object detection.

7. Decoupling Capacitor (Typically 100 nF)

Function:

Filters power supply noise.

Stabilizes the supply voltage.

Why it is used:

To improve the reliability and stability of the sensor module.

8. Output Connector

Pins:

VCC – Power supply (3.3 V to 5 V)

GND – Ground

OUT – Digital output signal

Why it is used:

To connect the sensor module to a microcontroller such as Arduino, ESP32, or STM32.

Working Principle

The IR LED continuously emits infrared light.

When an object is placed in front of the sensor, the emitted IR light is reflected.

The IR receiver detects the reflected IR light and converts it into an electrical signal.

The LM393 comparator compares this signal with the reference voltage set by the potentiometer.

If the reflected signal exceeds the threshold, the comparator changes the output state.

The indicator LED turns ON, and the OUT pin sends a digital HIGH or LOW signal (depending on the module design) to the microcontroller.

Applications of IR Obstacle Sensor Module

*Obstacle avoidance robots

*Line follower robots

*Automatic door systems

*Object detection systems

*Industrial automation

*Conveyor belt object detection

*Object counting systems

*Smart parking systems

*Security and intrusion detection systems

*Automatic lighting control

*Home automation systems

*Contactless switching applications