## Objective

- Identify all the components used in the LCD module.

- Understand the function of each component.

- Redraw the schematic in KiCad.

- Study the working principle of the LCD display.

- Analyze the PCB layout and signal flow.

# Main Components

## 1. LCD Panel (16×2 Character Display)

### Function

- Displays 16 characters in each row.

- Total 2 rows.

- Each character is formed using a 5×8 dot matrix.

### Why it is used

- To display text, numbers and symbols.

## 2. HD44780 LCD Controller IC (Main IC)

### Function

- Controls all LCD operations.

- Receives commands from the microcontroller.

- Stores display data in DDRAM.

- Stores custom characters in CGRAM.

- Generates timing signals.

- Controls LCD segments.

### Why it is used

- It acts as the brain of the LCD module.

## 3. LCD Driver IC (Sometimes Integrated)

### Function

- Drives COM and SEG lines.

- Applies AC voltage to LCD pixels.

- Prevents LCD damage.

### Why it is used

- To drive all LCD pixels correctly.

## 4. Character Generator ROM (CGROM)

### Function

- Stores ASCII characters.

- Converts binary codes into dot patterns.

### Why it is used

- To display letters and numbers.

## 5. Character Generator RAM (CGRAM)

### Function

- Stores custom characters.

- Maximum 8 user-defined characters.

### Why it is used

- Allows custom symbols.

## 6. Display Data RAM (DDRAM)

### Function

- Stores characters currently displayed.

### Why it is used

- Holds display memory.

## 7. LCD Glass

### Function

- Liquid crystal layer changes transparency.

- Displays characters.

### Why it is used

- Actual display medium.

## 8. Zebra Connector

### Function

- Connects PCB to LCD glass.

### Why it is used

- Flexible electrical connection.

## 9. Backlight LED

### Function

- Illuminates LCD.

### Why it is used

- Enables viewing in low light.

## 10. Current Limiting Resistor

### Function

- Limits current through backlight LED.

### Why it is used

- Protects LED.

## 11. Contrast Pin (VO)

### Function

- Controls LCD contrast.

### Why it is used

- Makes characters visible.

## 12. Potentiometer (External)

### Function

- Adjusts contrast voltage.

### Why it is used

- User can set proper display visibility.

## 13. Decoupling Capacitor (100 nF)

### Function

- Filters power supply noise.

### Why it is used

- Stable operation.

## 14. VCC Pin

### Function

- Supplies +5V.

### Why it is used

- Powers the module.

## 15. GND Pin

### Function

- Ground reference.

### Why it is used

- Completes the circuit.

## 16. RS Pin

### Function

- Register Select.

RS = 0 → Command Register

RS = 1 → Data Register

### Why it is used

- Selects command or data.

## 17. RW Pin

### Function

- Read / Write control.

RW = 0 → Write

RW = 1 → Read

### Why it is used

- Controls data direction.

## 18. Enable (E)

### Function

- Latches incoming data.

### Why it is used

- Executes commands.

## 19. Data Bus (D0–D7)

### Function

- Transfers commands and display data.

### Why it is used

- Communication interface.

## 20. Oscillator Circuit

### Function

- Generates internal clock.

### Why it is used

- Synchronizes LCD operation.

# Complete Pin Description

| Pin | Name | Function |
| --- | --- | --- |
| 1 | VSS | Ground |
| 2 | VDD | +5V Supply |
| 3 | VO | Contrast |
| 4 | RS | Register Select |
| 5 | RW | Read/Write |
| 6 | E | Enable |
| 7 | D0 | Data Bit 0 |
| 8 | D1 | Data Bit 1 |
| 9 | D2 | Data Bit 2 |
| 10 | D3 | Data Bit 3 |
| 11 | D4 | Data Bit 4 |
| 12 | D5 | Data Bit 5 |
| 13 | D6 | Data Bit 6 |
| 14 | D7 | Data Bit 7 |
| 15 | LED+ | Backlight Anode |
| 16 | LED− | Backlight Cathode |

# Working Principle

- +5V powers the LCD.

- Contrast voltage is applied to VO.

- Microcontroller sends commands.

- Commands enter HD44780 controller.

- Data is stored in DDRAM.

- Character codes are converted using CGROM.

- Driver IC energizes COM and SEG lines.

- LCD crystals rotate according to AC voltage.

- Character appears on screen.

- Backlight LEDs illuminate the display.

# Signal Flow

Microcontroller

↓

RS, RW, E

↓

HD44780 Controller

↓

DDRAM / CGROM / CGRAM

↓

LCD Driver

↓

COM & SEG Lines

↓

LCD Glass

↓

Visible Character

# Typical External Components

- 10 kΩ Potentiometer (Contrast)

- 220 Ω Resistor (Backlight LED)

- 100 nF Decoupling Capacitor

- 5V Supply

# Applications

- Embedded Systems

- Arduino Projects

- STM32 Projects

- ESP32 Projects

- Digital Clocks

- Home Automation

- Industrial Control Panels

- Medical Devices

- Measuring Instruments

- IoT Systems

# Advantages

- Low power consumption

- Easy microcontroller interfacing

- Low cost

- Reliable

- Supports 4-bit and 8-bit modes

- Easy to program