## Objective

- To identify all the components used in the MPU6050 sensor module.

- To understand the function of each component.

- To redraw the schematic in KiCad.

- To study the working principle of the module.

# Reverse Engineering of MPU6050 Sensor Module – Components and Their Functions

## 1. MPU6050 Sensor IC (Main Component)

### Function

- Measures acceleration along the X, Y, and Z axes.

- Measures angular velocity (rotation) along the X, Y, and Z axes.

- Contains a 3-axis accelerometer and a 3-axis gyroscope.

- Includes a Digital Motion Processor (DMP).

### Why it is used

- To detect motion, tilt, vibration, and orientation.

## 2. 3-Axis Accelerometer (Inside MPU6050 IC)

### Function

- Measures linear acceleration on X, Y, and Z axes.

### Why it is used

- To detect movement and tilt.

## 3. 3-Axis Gyroscope (Inside MPU6050 IC)

### Function

- Measures angular velocity on X, Y, and Z axes.

### Why it is used

- To detect rotation and orientation.

## 4. Digital Motion Processor (DMP)

### Function

- Processes motion data internally.

- Combines accelerometer and gyroscope data.

### Why it is used

- Reduces the processing load on the microcontroller.

## 5. Internal Temperature Sensor

### Function

- Measures the temperature of the sensor IC.

### Why it is used

- Used for internal calibration and temperature monitoring.

## 6. Voltage Regulator (Typically AMS1117-3.3V or Similar)

### Function

- Converts 5V input to 3.3V.

### Why it is used

- Powers the MPU6050 IC safely.

## 7. Pull-up Resistors (Typically 4.7 kΩ or 10 kΩ)

### Function

- Connected to the SDA and SCL lines.

- Keeps the I²C communication lines HIGH when idle.

### Why it is used

- Ensures reliable I²C communication.

## 8. Decoupling Capacitors (Typically 100 nF and 10 µF)

### Function

- Filters power supply noise.

- Stabilizes the supply voltage.

### Why it is used

- Improves sensor stability and accuracy.

## 9. Power Indicator LED (Available on Some Modules)

### Function

- Indicates that the module is powered.

### Why it is used

- Shows power status.

## 10. Header Pins

### Function

- Connects the module to external circuits.

### Why it is used

- Provides electrical connections.

# Pin Description

| Pin | Function |
| --- | --- |
| VCC | Power Supply (3.3V–5V depending on module) |
| GND | Ground |
| SCL | I²C Clock Line |
| SDA | I²C Data Line |
| XDA | Auxiliary I²C Data |
| XCL | Auxiliary I²C Clock |
| AD0 | I²C Address Selection |
| INT | Interrupt Output |

# Working Principle

- Power is supplied to the module.

- The voltage regulator provides a stable 3.3V to the MPU6050 IC.

- The accelerometer measures linear acceleration.

- The gyroscope measures angular velocity.

- The DMP processes and combines the sensor data.

- The processed data is stored in internal registers.

- The microcontroller reads the data through the I²C interface.

- The INT pin can notify the microcontroller when new data is ready.

# Signal Flow

Motion

↓

Accelerometer + Gyroscope

↓

Digital Motion Processor (DMP)

↓

Internal Registers

↓

I²C Interface (SDA & SCL)

↓

Microcontroller (Arduino / ESP32 / STM32)

# Typical External Components on the GY-521 Module

- MPU6050 Sensor IC

- AMS1117-3.3V Voltage Regulator (or equivalent)

- 2 × I²C Pull-up Resistors (4.7 kΩ or 10 kΩ)

- 100 nF Ceramic Capacitor

- 10 µF Electrolytic/Ceramic Capacitor

- Power LED (on some modules)

- Pin Header

# Applications

- Self-balancing Robots

- Drones and Quadcopters

- Gesture Recognition

- Motion Tracking

- Camera Stabilization (Gimbal)

- Fall Detection Systems

- Wearable Devices

- Gaming Controllers

- IoT Projects

- Embedded Systems

# Advantages

- 6 Degrees of Freedom (6-DoF)

- High accuracy motion sensing

- Built-in Digital Motion Processor (DMP)

- Low power consumption

- I²C communication interface

- Compact size

- Easy to interface with Arduino, ESP32, and STM32

- Low cost and widely available