## Objective

- To identify all the components used in the PIR sensor module.

- To understand the function of each component.

- To redraw the schematic in KiCad.

- To study the working principle of the module.

# Reverse Engineering of PIR Sensor Module – Components and Their Functions

## 1. PIR Sensor Element (Pyroelectric Infrared Sensor) – Main Component

### Function

- Detects changes in infrared (IR) radiation emitted by humans and animals.

- Generates a small electrical signal when motion is detected.

### Why it is used

- To detect human or animal movement without physical contact.

## 2. Fresnel Lens

### Function

- Focuses infrared radiation onto the PIR sensor element.

- Increases the sensing area and detection range.

### Why it is used

- Improves motion detection performance.

## 3. BISS0001 PIR Controller IC (Main IC)

### Function

- Amplifies the weak signal from the PIR sensor.

- Filters unwanted noise.

- Compares the signal with a threshold.

- Generates a digital HIGH or LOW output.

### Why it is used

- Processes the PIR sensor signal and controls the module output.

## 4. Voltage Regulator (Typically AMS1117-3.3V or Equivalent)

### Function

- Converts the input voltage to a stable operating voltage.

### Why it is used

- Protects and powers the PIR circuitry.

## 5. Timing Potentiometer

### Function

- Adjusts the output HIGH duration after motion detection.

### Why it is used

- Allows users to set how long the output remains active.

## 6. Sensitivity Potentiometer

### Function

- Adjusts the motion detection sensitivity and range.

### Why it is used

- Allows users to change the detection distance.

## 7. Signal Amplifier Circuit

### Function

- Amplifies the weak signal generated by the PIR sensor.

### Why it is used

- Makes the sensor output detectable by the controller IC.

## 8. Comparator Circuit

### Function

- Compares the amplified signal with the reference level.

### Why it is used

- Determines whether valid motion is present.

## 9. Decoupling Capacitors

### Function

- Filters power supply noise.

- Stabilizes the operating voltage.

### Why it is used

- Ensures reliable sensor operation.

## 10. Resistors

### Function

- Used for biasing, timing, pull-up, pull-down, and current limiting.

### Why it is used

- Ensures proper operation of the circuit.

## 11. Trigger Mode Jumper (H/L)

### Function

- Selects Repeat Trigger mode or Non-Repeat Trigger mode.

### Why it is used

- Allows different motion detection behaviors.

## 12. Output Driver Stage

### Function

- Drives the OUT pin with a digital HIGH or LOW signal.

### Why it is used

- Interfaces with external microcontrollers.

## 13. VCC Pin

### Function

- Supplies 5V–20V power to the module.

### Why it is used

- Powers the module.

## 14. GND Pin

### Function

- Provides the ground reference.

### Why it is used

- Completes the electrical circuit.

## 15. OUT Pin

### Function

- Outputs HIGH when motion is detected.

- Outputs LOW when no motion is detected.

### Why it is used

- Sends motion detection information to the microcontroller.

# Pin Description

| Pin | Function |
| --- | --- |
| VCC | Power Supply (5V–20V) |
| OUT | Digital Output |
| GND | Ground |

# Working Principle

- The Fresnel lens collects infrared radiation from the surroundings.

- The PIR sensor detects changes in IR energy caused by moving people or animals.

- The sensor generates a very small electrical signal.

- The amplifier increases the signal strength.

- The BISS0001 IC filters and processes the signal.

- If the signal exceeds the threshold, the IC sets the OUT pin HIGH.

- The timing potentiometer determines how long the output remains HIGH.

- The sensitivity potentiometer controls the detection range.

- After the delay time, the OUT pin returns LOW if no further motion is detected.

# Signal Flow

Human Motion

↓

Infrared Radiation

↓

Fresnel Lens

↓

PIR Sensor Element

↓

Signal Amplifier

↓

BISS0001 Controller IC

↓

Output Driver

↓

OUT Pin

↓

Arduino / ESP32 / STM32

# Typical Components on HC-SR501 Module

- PIR Sensor Element

- Fresnel Lens

- BISS0001 Controller IC

- AMS1117 Voltage Regulator (or equivalent)

- Timing Potentiometer

- Sensitivity Potentiometer

- Ceramic Capacitors

- Electrolytic Capacitors

- Resistors

- Trigger Mode Jumper

- Pin Header

# Applications

- Automatic Lighting Systems

- Motion-Activated Security Systems

- Smart Home Automation

- Burglar Alarms

- Automatic Doors

- Occupancy Detection

- Energy Saving Systems

- Robotics

- IoT Projects

- Embedded Systems

# Advantages

- Non-contact motion detection

- Low power consumption

- Wide detection angle (typically 120°)

- Detection range up to 7 meters (adjustable)

- Easy to interface with Arduino, ESP32, and STM32

- Low cost and reliable

- Adjustable sensitivity and delay time

- Suitable for indoor automation projects