## Objective

- To identify all the components used in the Rain Sensor module.

- To understand the function of each component.

- To redraw the schematic in KiCad.

- To study the working principle of the module.

# Reverse Engineering of Rain Sensor Module – Components and Their Functions

## 1. Rain Sensing Plate (Main Component)

### Function

- Detects the presence of water or rain droplets.

- The resistance between the copper traces decreases when water is present.

### Why it is used

- To detect rainfall or water on the sensor surface.

## 2. LM393 Comparator IC

### Function

- Compares the sensor voltage with a reference voltage.

- Generates a digital HIGH or LOW output.

### Why it is used

- Provides digital rain detection.

## 3. Preset Potentiometer (Typically 10 kΩ)

### Function

- Adjusts the reference voltage.

- Controls the rain detection sensitivity.

### Why it is used

- Allows the user to set the detection threshold.

## 4. Fixed Resistors

### Function

- Forms voltage divider circuits.

- Biases the comparator IC.

- Limits current where required.

### Why it is used

- Ensures proper circuit operation.

## 5. Power Indicator LED

### Function

- Indicates that the module is powered.

### Why it is used

- Shows power status.

## 6. Output Indicator LED

### Function

- Turns ON when rain is detected.

### Why it is used

- Indicates the output state.

## 7. LED Current Limiting Resistors

### Function

- Limits current through the LEDs.

### Why it is used

- Protects the LEDs from excessive current.

## 8. Decoupling Capacitor (Typically 100 nF)

### Function

- Filters power supply noise.

- Stabilizes the operating voltage.

### Why it is used

- Improves reliable operation.

## 9. Sensor Connector Header

### Function

- Connects the rain sensing plate to the comparator module.

### Why it is used

- Transfers the sensor signal.

## 10. VCC Pin

### Function

- Supplies 3.3V–5V power to the module.

### Why it is used

- Powers the module.

## 11. GND Pin

### Function

- Provides the ground reference.

### Why it is used

- Completes the electrical circuit.

## 12. AO (Analog Output)

### Function

- Outputs an analog voltage proportional to the amount of water detected.

### Why it is used

- Enables measurement of rain intensity.

## 13. DO (Digital Output)

### Function

- Outputs HIGH or LOW depending on the comparator result.

### Why it is used

- Enables digital rain detection.

# Pin Description

| Pin | Function |
| --- | --- |
| VCC | Power Supply (3.3V–5V) |
| GND | Ground |
| AO | Analog Output |
| DO | Digital Output |

# Working Principle

- The rain sensing plate is exposed to rain or water droplets.

- Water bridges the copper tracks, reducing their resistance.

- The voltage across the sensing plate changes.

- The analog voltage is available at the AO pin.

- The LM393 comparator compares this voltage with the threshold set by the potentiometer.

- If the threshold is exceeded, the DO pin changes state (HIGH or LOW depending on the module design).

- The output indicator LED turns ON to indicate rain detection.

# Signal Flow

Rain / Water Droplets

↓

Rain Sensing Plate

↓

Voltage Divider

↓

AO (Analog Output)

OR

Voltage Divider

↓

LM393 Comparator

↓

DO (Digital Output)

↓

Arduino / ESP32 / STM32

# Typical Components on the Rain Sensor Module

- Rain Sensing Plate (Copper PCB)

- LM393 Comparator IC

- 10 kΩ Potentiometer

- Fixed Resistors

- Power LED

- Output LED

- LED Current Limiting Resistors

- 100 nF Decoupling Capacitor

- Pin Header

- Sensor Connector

# Applications

- Automatic Wiper Systems

- Smart Irrigation Systems

- Weather Monitoring Stations

- Rain Alarm Systems

- Smart Windows

- Smart Home Automation

- Agriculture Monitoring

- IoT Projects

- Embedded Systems

- Robotics

# Advantages

- Low cost

- Easy to interface with Arduino, ESP32, and STM32

- Provides both analog and digital outputs

- Adjustable sensitivity

- Simple circuit design

- Low power consumption

- Suitable for weather monitoring and automation

- Reliable rain and water detection