## Objective

- To identify all the components used in the Soil Moisture Sensor module.

- To understand the function of each component.

- To redraw the schematic in KiCad.

- To study the working principle of the module.

# Reverse Engineering of Soil Moisture Sensor Module – Components and Their Functions

## 1. Soil Moisture Probe (Main Component)

### Function

- Detects the moisture content in soil.

- Measures the electrical conductivity between the two metal probes.

- Higher moisture results in lower resistance.

### Why it is used

- To measure the moisture level of the soil.

## 2. LM393 Comparator IC

### Function

- Compares the sensor voltage with a reference voltage.

- Produces a digital HIGH or LOW output.

### Why it is used

- Provides digital soil moisture detection.

## 3. Preset Potentiometer (Typically 10 kΩ)

### Function

- Adjusts the reference voltage.

- Controls the moisture detection threshold.

### Why it is used

- Allows users to set the desired moisture level.

## 4. Fixed Resistors

### Function

- Forms voltage divider circuits.

- Biases the comparator.

- Limits current where required.

### Why it is used

- Ensures proper circuit operation.

## 5. Power Indicator LED

### Function

- Indicates that the module is powered.

### Why it is used

- Shows power status.

## 6. Output Indicator LED

### Function

- Turns ON when the moisture level crosses the preset threshold.

### Why it is used

- Indicates the output status.

## 7. LED Current Limiting Resistors

### Function

- Limits current through the LEDs.

### Why it is used

- Protects the LEDs from excessive current.

## 8. Decoupling Capacitor (Typically 100 nF)

### Function

- Filters power supply noise.

- Stabilizes the supply voltage.

### Why it is used

- Improves reliable operation.

## 9. Probe Connector

### Function

- Connects the soil moisture probe to the comparator module.

### Why it is used

- Transfers the probe signal to the processing circuit.

## 10. VCC Pin

### Function

- Supplies 3.3V–5V power.

### Why it is used

- Powers the module.

## 11. GND Pin

### Function

- Provides the common ground reference.

### Why it is used

- Completes the electrical circuit.

## 12. AO (Analog Output)

### Function

- Outputs an analog voltage proportional to the soil moisture level.

### Why it is used

- Enables continuous moisture measurement.

## 13. DO (Digital Output)

### Function

- Outputs HIGH or LOW depending on the comparator result.

### Why it is used

- Enables threshold-based moisture detection.

# Pin Description

| Pin | Function |
| --- | --- |
| VCC | Power Supply (3.3V–5V) |
| GND | Ground |
| AO | Analog Output |
| DO | Digital Output |

# Working Principle

- The soil moisture probe is inserted into the soil.

- Moist soil conducts electricity better than dry soil.

- The resistance between the probe electrodes changes according to the moisture level.

- The probe generates a varying analog voltage.

- The AO pin provides this analog voltage to the microcontroller.

- The LM393 comparator compares the voltage with the threshold set by the potentiometer.

- If the threshold is crossed, the DO pin changes state (HIGH or LOW depending on the module design).

- The output LED indicates the moisture detection status.

# Signal Flow

Soil Moisture

↓

Soil Moisture Probe

↓

Voltage Divider

↓

AO (Analog Output)

OR

Voltage Divider

↓

LM393 Comparator

↓

DO (Digital Output)

↓

Arduino / ESP32 / STM32

# Typical Components on the Soil Moisture Sensor Module

- Soil Moisture Probe (YL-69 / FC-28)

- LM393 Comparator IC

- 10 kΩ Potentiometer

- Fixed Resistors

- Power LED

- Output LED

- LED Current Limiting Resistors

- 100 nF Decoupling Capacitor

- Pin Header

- Probe Connector

# Applications

- Smart Irrigation Systems

- Precision Agriculture

- Automatic Plant Watering

- Greenhouse Monitoring

- Garden Automation

- Soil Condition Monitoring

- IoT Agriculture Projects

- Embedded Systems

- Environmental Monitoring

- Robotics

# Advantages

- Low cost

- Easy to interface with Arduino, ESP32, and STM32

- Provides both analog and digital outputs

- Adjustable sensitivity

- Simple circuit design

- Low power consumption

- Suitable for smart farming and irrigation systems

- Real-time soil moisture monitoring