Objective

To identify all the components used in the DHT11 sensor module.

To understand the function of each component.

To redraw the schematic in KiCad.

To study the working principle of the module.

Reverse Engineering of DHT11 Sensor Module – Components and Their Functions

1. DHT11 Sensor IC (Main Component)

Function:

- Measures temperature and relative humidity.

- Contains a humidity sensing element, NTC thermistor, ADC, calibration memory, and an internal microcontroller.

- Sends the measured data as digital output through the DATA pin.

Why it is used:

- To measure both temperature and humidity using a single sensor.

---

2. Humidity Sensing Element (Inside the DHT11 IC)

Function:

- Measures the relative humidity of the surrounding air.

- The capacitance changes according to the humidity level.

Why it is used:

- To accurately detect humidity.

---

3. NTC Thermistor (Inside the DHT11 IC)

Function:

- Measures the ambient temperature.

- Its resistance decreases as the temperature increases.

Why it is used:

- To measure the surrounding temperature.

---

4. Internal Microcontroller / ASIC (Inside the DHT11 IC)

Function:

- Reads the signals from the humidity sensor and thermistor.

- Converts the analog signals into digital data using an ADC.

- Performs calibration and sends the processed digital data through the DATA pin.

Why it is used:

- To provide accurate and calibrated digital output.

---

5. Pull-up Resistor (Typically 10 kΩ)

Function:

- Connected between VCC and the DATA pin.

- Keeps the DATA line in a HIGH state when idle.

- Ensures stable communication with the microcontroller.

Why it is used:

- To maintain reliable digital communication.

---

6. Decoupling Capacitor (Typically 100 nF)

Function:

- Connected between VCC and GND.

- Filters power supply noise.

- Provides a stable supply voltage to the sensor.

Why it is used:

- To improve power stability and prevent incorrect sensor readings.

---

7. VCC Pin

Function:

- Supplies 3.3 V to 5 V power to the DHT11 sensor.

Why it is used:

- To power the sensor.

---

8. DATA Pin

Function:

- Transmits temperature and humidity data to the microcontroller.

Why it is used:

- To enable digital communication with the host controller.

---

9. GND Pin

Function:

- Provides the common ground reference for the circuit.

Why it is used:

- To complete the electrical circuit.

Applications

Weather monitoring systems

Smart home automation

Greenhouse monitoring

HVAC (Heating, Ventilation, and Air Conditioning)

Indoor environmental monitoring

IoT-based temperature and humidity monitoring projects

Data logging systems

Advantages

Measures both temperature and humidity using a single sensor.

Digital output (no external ADC required).

Low power consumption.

Easy to interface with microcontrollers.

Low cost and widely available.